| user nick | Panda 500 | Panda 200 | Lotty 600 | Lotty 400 | Panda 166 |
| --- | --- | --- | --- | --- | --- |
| CPU name | Intel Pentium 2 Celeron, 500 MHz | Intel Pentium Pro-S, 200 MHz | Intel Pentium 3, 600 MHz | Intel Pentium 2, 400 MHz | Intel Pentium MMX, 166 MHz |
| motherboard | ZX98-CU | W282 | AOpen AX6BC | AOpen AX6BC | - |
| chipset | Intel 82443ZX | - | 440BX | 440BX | VIA VT82C586A |
| CPU type | Pentium 2 | Pentium Pro | Pentium 3 | Pentium 2 | Pentium 1 |
| Vendor | Intel | Intel | Intel | Intel | Intel |
| Family | 6 | 6 | 6 | 6 | 5 |
| Model | 6 | 1 | 7 | 5 | 4 |
| Frequency | 501.146 MHz | 199.794 MHz | 601.381 MHz | 400.920 MHz | 167.049 MHz |
| MMX | yes | - | yes | yes | yes |
| SSE | - | - | yes | - | - |
| SSE2 | - | - | - | - | - |
| 3DNow! | - | - | - | - | - |
| 3DNow! Ext | - | - | - | - | - |
| MMX+ AMD | - | - | - | - | - |
| MMX+ Cyrix | - | - | - | - | - |
| FPU | yes | yes | yes | yes | yes |
| TSC | yes | yes | yes | yes | yes |
| mov reg8,N | 11.7 | 38.4 | 9.5 | 9.3 | 32.7 |
| mov reg16,N | 20.0 | 72.2 | 16.8 | 17.3 | 70.8 |
| mov reg32,N | 25.8 | 90.2 | 24.3 | 23.2 | 85.5 |
| mov reg32,reg32 | 9.6 | 34.2 | 7.8 | 8.1 | 23.2 |
| inc reg32 | 4.9 | 6.5 | 3.2 | 3.3 | 9.8 |
| dec reg32 | 4.6 | 13.6 | 4.0 | 4.7 | 10.8 |
| mov reg32,[addr+offset] | 53.3 | 206 | 41.3 | 50.4 | 171 |
| mov [addr+offset],reg32 | 43.5 | 210 | 39.9 | 49.4 | 226 |
| mov reg32,[addr+offset+1] | 78.0 | 291 | 67.0 | 74.2 | 241 |
| mov [addr+offset+1],reg32 | 68.0 | 290 | 60.8 | 66.5 | 284 |
| xchg reg32,reg32 | 7.5 | 15.7 | 3.2 | 4.2 | 11.6 |
| add reg32,reg32 | 11.7 | 33.1 | 7.7 | 7.7 | 29.3 |
| mul reg32 | 13.4 | 36.1 | 6.6 | 7.5 | 23.4 |
| div reg32 | 13.4 | 22.9 | 18.8 | 55.4 | 111 |
| push reg32 | 5.6 | 15.6 | 4.8 | 4.7 | 80.3 |
| pop reg32 | 35.5 | 90.4 | 25.6 | 27.1 | 78.4 |
| pusha | 147 | 593 | 131 | 142 | 479 |
| popa | 171 | 597 | 147 | 153 | 790 |
| jmp short | 14.0 | 32.3 | 10.4 | 16.9 | 44.2 |
| jmp near | 32.4 | 84.9 | 22.8 | 24.3 | 59.7 |
| jc short (jump) | 17.7 | 44.2 | 14.2 | 21.7 | 44.5 |
| jc short (not jump) | 10.1 | 14.8 | 7.6 | 7.5 | 21.7 |
| loop | 31.0 | 77.1 | 22.0 | 38.5 | 67.6 |
| nop | 3.8 | 5.6 | 3.4 | 3.2 | 10.4 |
| call addr | 41.7 | 101 | 34.7 | 33.9 | 161 |
| ret | 23.8 | 59.2 | 19.2 | 29.2 | 46.2 |
| out N,al | 1393 | 1455 | 1521 | 1554 | 1138 |
| in al,N | 668 | 710 | 570 | 605 | 1146 |
| out dx,al | 1001 | 1009 | 992 | 1008 | 1117 |
| in al,dx | 872 | 902 | 918 | 955 | 1116 |
| rdtsc | 655 | 158 | 52.2 | 80.0 | 47.1 |
| fnsave | 584 | 2024 | 513 | 597 | 1727 |
| fsave | 635 | 1783 | 592 | 672 | 1938 |
| frstor | 1051 | 2384 | 843 | 914 | 1827 |
| fxsave | 819 | - | 705 | 810 | - |
| fxrstor | 909 | - | 739 | 804 | - |
| memfill stosd (4KB) | 19005 | 26937 | 13818 | 16684 | 62138 |
| memcopy movsd (4KB) | 40964 | 101394 | 28872 | 30824 | 150471 |
| memfill MMX0 (4KB) | 21378 | - | 15997 | 15529 | 31650 |
| memcopy MMX0 (4KB) | 35070 | - | 29904 | 33920 | 121109 |
| memfill MMX0..7 (4KB) | 18448 | - | 17452 | 19595 | 31189 |
| memcopy MMX0..7 (4KB) | 39013 | - | 32342 | 30647 | 121330 |
| total time (without In/Out) | 1204 us | 3455 us | 982 us | 1151 us | 3256 us |

instruction times are in nanoseconds